Hardware Specification Document

Project: Fari Sensor Hub (Rev A)

Author: Zhixian Zuo

1. Functional Requirements

The Fari Sensor Hub acts as the centralized environmental and motion perception node for the Fari eldercare assistant. Mounted on the robot's chest, the board must reliably capture and process local sensor data before relaying it to the main compute unit.

- Core Microcontroller: Espressif ESP32-S3.

- Sensor Stack:

  - 6-axis IMU for motion and tilt detection.

  - VL53L1X Time-of-Flight (ToF) ranger for proximity and obstacle detection.

  - Ambient temperature and humidity sensor.

- Peripheral Management: I2C expander/multiplexer to manage address conflicts and bus capacitance.

- Communication Interface: UART interface for stable, low-latency data transmission to the main compute node.

2. Electrical Requirements

Given the stringent requirements for signal integrity and reliable physical operation in service robotics, the electrical architecture must prioritize robust power delivery and fault protection.

- Power Input: USB-C receptacle providing 5 V (VBUS).

- System Power Rail: Down-converted to a highly stable 3.3 V DC rail using an LDO or buck converter to power the MCU and all sensor logic.

- Protection Circuitry:

  - ESD protection diodes array on the USB-C data and power lines.

  - PTC Polyfuse on the VBUS line to prevent overcurrent events and protect the upstream power source.

- Signal Integrity & Decoupling: Dedicated decoupling capacitors (e.g., 100nF || 1µF) placed intimately close to the power pins of every IC to minimize ground bounce and EMI.

- Testability: Clearly labeled physical test points (TPs) for 5V, 3.3V, GND, UART (TX/RX), and I2C (SCL/SDA) to facilitate bring-up and debugging.

3. Mechanical Requirements

The physical layout must adhere to strict dimensional constraints to integrate seamlessly into Fari's chest cavity.

- Form Factor: Strictly ≤ 40 mm × 40 mm rectangular footprint.

- Layer Stackup: 2-layer rigid PCB (FR4).

- Mounting: Four M2 or M2.5 mounting holes positioned near the corners for secure mechanical fastening.

- Component Placement: The USB-C connector must be placed precisely on the board edge. The VL53L1X ToF sensor must have an unobstructed line of sight (no tall components placed immediately adjacent to its field of view).

4. Environmental Requirements

As Fari is an indoor eldercare assistant, the board will operate in benign environments but must maintain safe touch temperatures and absolute silence.

- Operating Temperature: 0°C to 45°C (standard indoor ambient).

- Thermal Management: Passive cooling only (natural convection). Component placement and copper pours must be optimized to dissipate heat from the ESP32-S3 and power regulator without the use of active fans, adhering to Fari's zero-noise requirement.

- Humidity: 10% to 90% non-condensing.